## 1. main.c

### Main application — sensor init, buzzer/LED control, pulse monitoring loop

#include<lpc21xx.h>

#include <stdio.h>

#include"header.h"

#define grn (1<<16)

#define rd (1<<17)

#define buz (1<<18)

#define max_temp 38

#define min_heart_rate 60

#define max_heart_rate 99

#define min_spo2 90

#define ir_threshold 75000

MAX30102_DATA SAMPLE;

pulse_result PULSE;

int main()

{

//float cur_temp =0.0,v;

char s[50],s2[20];

//unsigned char data[6];

//int cur_heart_rate=0;

//int cur_spo2=0;

//unsigned int id;

IODIR0=buz||grn||rd;

uart0_init(9600);

adc_init();

lcd_init();

i2c_init();

//DHT11_Start();

pulse_init();

sprintf(s,"MODE = %0x2x FIFO=%02x SPO2=%02x LED1=%02x LED2=%02x\r\n",Pulse_ReadReg(MAX_MODE_CONFIG),Pulse_ReadReg(MAX_FIFO_CONFIG),

Pulse_ReadReg(MAX_SPO2_CONFIG),Pulse_ReadReg(MAX_LED1_PA),Pulse_ReadReg(MAX_LED2_PA));

uart0_tx_str(s);

algorithm_init();

//lm35 sensor

while(1){

//DHT11_ReemadTperature();

//   delay_ms(100);

////a=adc_read(1);

////v=((a*3.3)/1023);

////cur_temp= (v-0.5)/0.01;

////

////sprintf(s," adc = %d volt = %.3f Temp = %.2f\r\n",a,v,cur_temp);

//uart0_float(cur_temp);

//uart0_tx_str(s);

Pulse_ReadFIFO(&SAMPLE);

//sprintf(s2,"RED = %d  IR = %d\r\n",SAMPLE.red,SAMPLE.ir);

//uart0_tx_str(s2);

//delay_ms(50);

if(SAMPLE.ir > ir_threshold){

if(algorithm_process(SAMPLE.ir,SAMPLE.red,&PULSE)){

sprintf(s,"bpm = %u  spo2 = %.1f\r\n",PULSE.bpm,PULSE.spo2);

uart0_tx_str(s);

delay_ms(1000);

}

}

}

}

## 2. Pulse Sensor Register Driver

### MAX30102 register read/write wrapper and FIFO sample reader

#include "header.h"

void Pulse_WriteReg(unsigned char reg,unsigned char value){

i2c_send(MAX30102_ADDR,reg,value);

}

unsigned char Pulse_ReadReg(unsigned char reg){

return i2c_read(MAX30102_ADDR, reg);

}

void pulse_init(void){

/*reset*/

Pulse_WriteReg(MAX_MODE_CONFIG,0x40);

delay_ms(100);

/*FIFO*/

Pulse_WriteReg(MAX_FIFO_WR_PTR,0x00);

Pulse_WriteReg(MAX_OVF_COUNTER,0x00);

Pulse_WriteReg(MAX_FIFO_RD_PTR,0x00);

/*FIFO CONFIG*/

Pulse_WriteReg(MAX_FIFO_CONFIG,0X0F);

/*SPO2 CONFIG*/

Pulse_WriteReg(MAX_SPO2_CONFIG,0X27);

/*SPO2 MODE*/

Pulse_WriteReg(MAX_MODE_CONFIG,0X03);

/*LED_CURRENT*/

Pulse_WriteReg(MAX_LED1_PA,0X24);

Pulse_WriteReg(MAX_LED2_PA,0X24);

/*INTERRUPT*/

Pulse_WriteReg(MAX_INTR_ENABLE_1,0X40);

Pulse_WriteReg(MAX_INTR_ENABLE_2,0X00);

}

void Pulse_ReadFIFO(MAX30102_DATA *sample){

unsigned char data[6];

i2c_read_multi(MAX30102_ADDR,MAX_FIFO_DATA,data,6);

//data = i2c_read(MAX30102_ADDR,MAX_FIFO_DATA);

sample->red = ((unsigned long )data[0] << 16) | ((unsigned long)data[1] << 8) | ((unsigned long)data[2]);

sample->ir = ((unsigned long )data[3] << 16) | ((unsigned long )data[4] << 8) | ((unsigned long )data[5]);

/*18 bit adc value*/

sample->red &= 0x03ffff;

sample->ir &= 0x03ffff;

}

## 3. I2C Driver

### I2C master init, single-byte write/read, and multi-byte read routines

#include "header.h"

void i2c_init(void){

PINSEL0|=0x50;//P0.2->SCL,P0.3->SDA

I2SCLH=I2SCLL=75;//100Kbps freq

/*select master mode*/

I2CONCLR=(1<<2);//AA=0

I2CONSET=(1<<6);//I2EN=1

}

/*i2c byte write frame*/

#define SI ((I2CONSET>>3)&1)

void i2c_write(u8 sa,u8 mr,u8 data){

/*step1:generate start condi*/

I2CONSET=(1<<5);//STA=1

I2CONCLR=(1<<3);//Clear SI*

while(SI==0);//waiting for start condi

I2CONCLR=(1<<5);//STA=0*

/*step2:send sa+w & check ack*/

I2DAT=sa;//send sa+w

I2CONCLR=(1<<3);//SI=0

while(SI==0);

if(I2STAT==0x20){

uart0_str("err:sa+w\r\n");

goto exit;

}

/*step3:send mr_addr & check ack*/

I2DAT=mr;//send mr_addr

I2CONCLR=(1<<3);//SI=0

while(SI==0);

if(I2STAT==0x30){

uart0_str("err:mr_addr\r\n");

goto exit;

}

/*step4:send data & check ack*/

I2DAT=data;//send data

I2CONCLR=(1<<3);//SI=0

while(SI==0);

if(I2STAT==0x30){

uart0_str("err:data\r\n");

goto exit;

}

/*step5:generate stop condi*/

exit:

I2CONSET=(1<<4);//STO=1

I2CONCLR=(1<<3);//Clear SI*

}

/*i2c byte read frame*/

u8 i2c_read(u8 sa, u8 mr){

u8 res=0;

/*step1:generate start condi*/

I2CONSET=(1<<5);

I2CONCLR=(1<<3);

while(SI==0);

I2CONCLR=(1<<5);

/*step2:send sa+w & check ack*/

I2DAT=sa^1;//send sa+w

I2CONCLR=(1<<3);//SI=0

while(SI==0);

if(I2STAT==0x20){

uart0_str("err:sa+w\r\n");

goto exit;

}

/*step3:send mr_addr & check ack*/

I2DAT=mr;//send mr_addr

I2CONCLR=(1<<3);//SI=0

while(SI==0);

if(I2STAT==0x30){

uart0_str("err:mr_addr\r\n");

goto exit;

}

/*step4:generate restart condi*/

I2CONSET=(1<<5);

I2CONCLR=(1<<3);

while(SI==0);

I2CONCLR=(1<<5);

/*step5:send sa+r & check ack*/

I2DAT=sa;//send SA+R

I2CONCLR=(1<<3);//SI=0

while(SI==0);

if(I2STAT==0x48){

uart0_str("err:sa+r\r\n");

goto exit;

}

/*step6:read data from slave & send noack*/

I2CONCLR=(1<<3);//SI=0

while(SI==0);//waiting for data receive

res=I2DAT;

/*step7:generate stop condi*/

exit:

I2CONSET=(1<<4);//STO=1

I2CONCLR=(1<<3);//SI=0

/*step8:return data*/

return res;

}

void i2c_mult_read(u8 sa_add, u8 mr_add, u8 len, u8 *buf)

{

u8 i;

/* START */

I2CONSET = (1<<5);

while(SI==0);

I2CONCLR = (1<<5)|(1<<3);

/* Slave Address + Write */

I2DAT = sa_add;

while(SI==0);

I2CONCLR = (1<<3);

if(I2STAT==0x20)

{

//  uart0_str("Slave Address Error\n");

goto exit;

}

/* Memory Address */

I2DAT = mr_add;

while(SI==0);

I2CONCLR = (1<<3);

if(I2STAT==0x30)

{

// uart0_str("Memory Address Error\n");

goto exit;

}

/* Repeated START */

I2CONSET = (1<<5);

while(SI==0);

I2CONCLR = (1<<5)|(1<<3);

/* Slave Address + Read */

I2DAT = sa_add^1;

while(SI==0);

I2CONCLR = (1<<3);

if(I2STAT==0x48)

{

// uart0_str("Read Address Error\n");

goto exit;

}

for(i=0;i<len;i++)

{

if(i < (len-1)) I2CONSET = (1<<2);      // ACK

else  I2CONCLR = (1<<2);      // NACK

while(SI==0);

buf[i] = I2DAT;

I2CONCLR = (1<<3);

if((i < (len-1)) && (I2STAT!=0x50))

break;

if((i==(len-1)) && (I2STAT!=0x58))

break;

}

exit:

I2CONSET = (1<<4);      // STOP

I2CONCLR = (1<<3);

}

## 4. Heart Rate / SpO2 Algorithm (Moving Average Method)

### Moving-average filter based peak detection for BPM and SpO2 estimation

#include "header.h"

#include"pulse.h"

#define SAMPLE_RATE 100

#define BUFFER_SIZE 100

#define FILTER_SIZE 8

#define MIN_PEAK_DISTANCE 40

#define MAX_BEATS 5

static unsigned long ir_filter[FILTER_SIZE];

static unsigned char filter_index;

static unsigned long ir_buffer[FILTER_SIZE];

static unsigned long red_buffer[BUFFER_SIZE];

static unsigned short sample_index;

static unsigned long prev_avg = 0;

static unsigned short last_peak =0;

static unsigned char rising =0;

static unsigned short beat_interval[MAX_BEATS];

static unsigned char beat_index;

static unsigned long moving_average(unsigned long value)

{

unsigned long sum=0;

unsigned char i;

ir_filter[filter_index]=value;

filter_index++;

if(filter_index>=FILTER_SIZE)

filter_index=0;

for(i=0;i<FILTER_SIZE;i++)

sum+=ir_filter[i];

return sum/FILTER_SIZE ;

}

void algorithm_init(){

unsigned short i;

for(i=0 ; i<	FILTER_SIZE ; i++)

ir_filter[i] = 0;

for(i=0 ; i<BUFFER_SIZE ; i++)

{

ir_buffer[i] = 0;

red_buffer[i]=0;

}

filter_index =0;

sample_index=0;

prev_avg =0;

beat_index =0;

last_peak	 =0;

rising =0;

}

unsigned char algorithm_process(unsigned long ir, unsigned long red,pulse_result *result){

unsigned short i;

unsigned long avg ;

unsigned short diff;

unsigned long ir_max,ir_min;

unsigned long red_max,red_min;

float ac_ir;

float dc_ir;

float ac_red;

float dc_red;

float ratio;

ir_buffer[sample_index] = ir;

red_buffer[sample_index]=red;

avg=moving_average(ir);

sample_index++;

if(sample_index >=  BUFFER_SIZE)

sample_index =0;

if(avg>prev_avg)

rising=1;

if((avg<prev_avg)&&rising)

{

diff=(sample_index>=last_peak)?(sample_index-last_peak):(BUFFER_SIZE-last_peak+sample_index);

if(diff>MIN_PEAK_DISTANCE)

{

beat_interval[beat_index]=diff;

beat_index++;

if(beat_index>=MAX_BEATS)

beat_index=0;

last_peak=sample_index;

rising=0;

diff=0;

for(i=0;i<MAX_BEATS;i++)

diff+=beat_interval[i];

diff/=MAX_BEATS;

result->bpm=(60*SAMPLE_RATE)/diff;

ir_max=0;

ir_min=0xffffffff;

red_max=0;

red_min=0xfffffff;

for(i=0 ; i< BUFFER_SIZE ; i++)

{

if(ir_buffer[i]>ir_max)

ir_max=ir_buffer[i];

if(ir_buffer[i]<ir_min)

ir_min=ir_buffer[i];

if(red_buffer[i]>red_max)

red_max=red_buffer[i];

if(red_buffer[i]<red_min)

red_min=red_buffer[i];

}

dc_ir=(ir_max+ir_min)/2.0f;

ac_ir=(ir_max-ir_min);

dc_red=(red_max+red_min)/2.0f;

ac_ir=(red_max-red_min);

ratio=(ac_red/dc_red)/(ac_ir/dc_ir);

result->spo2=110.0f-(25.0f*ratio);

if(result->spo2>100)

result->spo2=100;

if(result->spo2<80)

result->spo2=80;

result->valid=1;

prev_avg=avg;

return 1;

}

rising=0;

}

prev_avg=avg;

result->valid=0;

return 0;

}

## 5. Heart Rate & SpO2 Algorithm

### Autocorrelation-based periodicity detection with linear-trend removal for HR/SpO2

#include"header.h"

void rf_heart_rate_and_oxygen_saturation(u32 *pun_ir_buffer, s32 n_ir_buffer_length, u32 *pun_red_buffer, float *pn_spo2, s8 *pch_spo2_valid, s32 *pn_heart_rate,

s8 *pch_hr_valid, float *ratio, float *correl)

/**

* \brief        Calculate the heart rate and SpO2 level, Robert Fraczkiewicz version

* \par          Details

*               By detecting  peaks of PPG cycle and corresponding AC/DC of red/infra-red signal, the xy_ratio for the SPO2 is computed.

*

* \param[in]    *pun_ir_buffer           - IR sensor data buffer

* \param[in]    n_ir_buffer_length      - IR sensor data buffer length

* \param[in]    *pun_red_buffer          - Red sensor data buffer

* \param[out]    *pn_spo2                - Calculated SpO2 value

* \param[out]    *pch_spo2_valid         - 1 if the calculated SpO2 value is valid

* \param[out]    *pn_heart_rate          - Calculated heart rate value

* \param[out]    *pch_hr_valid           - 1 if the calculated heart rate value is valid

*

* \retval       None

*/

{

s32 k;

static s32 n_last_peak_interval=LOWEST_PERIOD;

float f_ir_mean,f_red_mean,f_ir_sumsq,f_red_sumsq;

float f_y_ac, f_x_ac, xy_ratio;

float beta_ir, beta_red, x;

float an_x[BUFFER_SIZE], *ptr_x; //ir

float an_y[BUFFER_SIZE], *ptr_y; //red

// calculates DC mean and subtracts DC from ir and red

f_ir_mean=0.0;

f_red_mean=0.0;

for (k=0; k<n_ir_buffer_length; ++k) {

f_ir_mean += pun_ir_buffer[k];

f_red_mean += pun_red_buffer[k];

}

f_ir_mean=f_ir_mean/n_ir_buffer_length ;

f_red_mean=f_red_mean/n_ir_buffer_length ;

// remove DC

for (k=0,ptr_x=an_x,ptr_y=an_y; k<n_ir_buffer_length; ++k,++ptr_x,++ptr_y) {

*ptr_x = pun_ir_buffer[k] - f_ir_mean;

*ptr_y = pun_red_buffer[k] - f_red_mean;

}

// RF, remove linear trend (baseline leveling)

beta_ir = rf_linear_regression_beta(an_x, mean_X, sum_X2);

beta_red = rf_linear_regression_beta(an_y, mean_X, sum_X2);

for(k=0,x=-mean_X,ptr_x=an_x,ptr_y=an_y; k<n_ir_buffer_length; ++k,++x,++ptr_x,++ptr_y) {

*ptr_x -= beta_ir*x;

*ptr_y -= beta_red*x;

}

// For SpO2 calculate RMS of both AC signals. In addition, pulse detector needs raw sum of squares for IR

f_y_ac=rf_rms(an_y,n_ir_buffer_length,&f_red_sumsq);

f_x_ac=rf_rms(an_x,n_ir_buffer_length,&f_ir_sumsq);

// Calculate Pearson correlation between red and IR

*correl=rf_Pcorrelation(an_x, an_y, n_ir_buffer_length)/sqrt(f_red_sumsq*f_ir_sumsq);

// Find signal periodicity

if(*correl>=min_pearson_correlation) {

// At the beginning of oximetry run the exact range of heart rate is unknown. This may lead to wrong rate if the next call does not find the _first_

// peak of the autocorrelation function. E.g., second peak would yield only 50% of the true rate.

if(LOWEST_PERIOD==n_last_peak_interval)

rf_initialize_periodicity_search(an_x, BUFFER_SIZE, &n_last_peak_interval, HIGHEST_PERIOD, min_autocorrelation_ratio, f_ir_sumsq);

// RF, If correlation os good, then find average periodicity of the IR signal. If aperiodic, return periodicity of 0

if(n_last_peak_interval!=0)

rf_signal_periodicity(an_x, BUFFER_SIZE, &n_last_peak_interval, LOWEST_PERIOD, HIGHEST_PERIOD, min_autocorrelation_ratio, f_ir_sumsq, ratio);

} else n_last_peak_interval=0;

// Calculate heart rate if periodicity detector was successful. Otherwise, reset peak interval to its initial value and report error.

if(n_last_peak_interval!=0) {

*pn_heart_rate = (s32)(FS60/n_last_peak_interval);

*pch_hr_valid  = 1;

} else {

n_last_peak_interval=LOWEST_PERIOD;

*pn_heart_rate = -999; // unable to calculate because signal looks aperiodic

*pch_hr_valid  = 0;

*pn_spo2 =  -999 ; // do not use SPO2 from this corrupt signal

*pch_spo2_valid  = 0;

return;

}

// After trend removal, the mean represents DC level

xy_ratio= (f_y_ac*f_ir_mean)/(f_x_ac*f_red_mean);  //formula is (f_y_ac*f_x_dc) / (f_x_ac*f_y_dc) ;

if(xy_ratio>0.02 && xy_ratio<1.84) { // Check boundaries of applicability

*pn_spo2 = (-45.060*xy_ratio + 30.354)*xy_ratio + 94.845;

*pch_spo2_valid = 1;

} else {

*pn_spo2 =  -999 ; // do not use SPO2 since signal an_ratio is out of range

*pch_spo2_valid  = 0;

}

}

float rf_linear_regression_beta(float *pn_x, float xmean, float sum_x2)

/**

* \brief        Coefficient beta of linear regression

* \par          Details

*               Compute directional coefficient, beta, of a linear regression of pn_x against mean-centered

*               point index values (0 to BUFFER_SIZE-1). xmean must equal to (BUFFER_SIZE-1)/2! sum_x2 is

*               the sum of squares of the mean-centered index values.

*               Robert Fraczkiewicz, 12/22/2017

* \retval       Beta

*/

{

float x,beta,*pn_ptr;

beta=0.0;

for(x=-xmean,pn_ptr=pn_x;x<=xmean;++x,++pn_ptr)

beta+=x*(*pn_ptr);

return beta/sum_x2;

}

float rf_autocorrelation(float *pn_x, s32 n_size, s32 n_lag)

/**

* \brief        Autocorrelation function

* \par          Details

*               Compute autocorrelation sequence's n_lag's element for a given series pn_x

*               Robert Fraczkiewicz, 12/21/2017

* \retval       Autocorrelation sum

*/

{

int16_t i, n_temp=n_size-n_lag;

float sum=0.0,*pn_ptr;

if(n_temp<=0) return sum;

for (i=0,pn_ptr=pn_x; i<n_temp; ++i,++pn_ptr) {

sum += (*pn_ptr)*(*(pn_ptr+n_lag));

}

return sum/n_temp;

}

void rf_initialize_periodicity_search(float *pn_x, s32 n_size, s32 *p_last_periodicity, s32 n_max_distance, float min_aut_ratio, float aut_lag0)

/**

* \brief        Search the range of true signal periodicity

* \par          Details

*               Determine the range of current heart rate by locating neighborhood of

*               the _first_ peak of the autocorrelation function. If at all lags until

*               n_max_distance the autocorrelation is less than min_aut_ratio fraction

*               of the autocorrelation at lag=0, then the input signal is insufficiently

*               periodic and probably indicates motion artifacts.

*               Robert Fraczkiewicz, 04/25/2020

* \retval       Average distance between peaks

*/

{

s32 n_lag;

float aut,aut_right;

// At this point, *p_last_periodicity = LOWEST_PERIOD. Start walking to the right,

// two steps at a time, until lag ratio fulfills quality criteria or HIGHEST_PERIOD

// is reached.

n_lag=*p_last_periodicity;

aut_right=aut=rf_autocorrelation(pn_x, n_size, n_lag);

// Check sanity

if(aut/aut_lag0 >= min_aut_ratio) {

// Either quality criterion, min_aut_ratio, is too low, or heart rate is too high.

// Are we on autocorrelation's downward slope? If yes, continue to a local minimum.

// If not, continue to the next block.

do {

aut=aut_right;

n_lag+=2;

aut_right=rf_autocorrelation(pn_x, n_size, n_lag);

} while(aut_right/aut_lag0 >= min_aut_ratio && aut_right<aut && n_lag<=n_max_distance);

if(n_lag>n_max_distance) {

// This should never happen, but if does return failure

*p_last_periodicity=0;

return;

}

aut=aut_right;

}

// Walk to the right.

do {

aut=aut_right;

n_lag+=2;

aut_right=rf_autocorrelation(pn_x, n_size, n_lag);

} while(aut_right/aut_lag0 < min_aut_ratio && n_lag<=n_max_distance);

if(n_lag>n_max_distance) {

// This should never happen, but if does return failure

*p_last_periodicity=0;

} else

*p_last_periodicity=n_lag;

}

void rf_signal_periodicity(float *pn_x, s32 n_size, s32 *p_last_periodicity, s32 n_min_distance, s32 n_max_distance, float min_aut_ratio, float aut_lag0, float *ratio)

/**

* \brief        Signal periodicity

* \par          Details

*               Finds periodicity of the IR signal which can be used to calculate heart rate.

*               Makes use of the autocorrelation function. If peak autocorrelation is less

*               than min_aut_ratio fraction of the autocorrelation at lag=0, then the input

*               signal is insufficiently periodic and probably indicates motion artifacts.

*               Robert Fraczkiewicz, 01/07/2018

* \retval       Average distance between peaks

*/

{

s32 n_lag;

float aut,aut_left,aut_right,aut_save;

bool left_limit_reached=false;

// Start from the last periodicity computing the corresponding autocorrelation

n_lag=*p_last_periodicity;

aut_save=aut=rf_autocorrelation(pn_x, n_size, n_lag);

// Is autocorrelation one lag to the left greater?

aut_left=aut;

do {

aut=aut_left;

n_lag--;

aut_left=rf_autocorrelation(pn_x, n_size, n_lag);

} while(aut_left>aut && n_lag>=n_min_distance);

// Restore lag of the highest aut

if(n_lag<n_min_distance) {

left_limit_reached=true;

n_lag=*p_last_periodicity;

aut=aut_save;

} else n_lag++;

if(n_lag==*p_last_periodicity) {

// Trip to the left made no progress. Walk to the right.

aut_right=aut;

do {

aut=aut_right;

n_lag++;

aut_right=rf_autocorrelation(pn_x, n_size, n_lag);

} while(aut_right>aut && n_lag<=n_max_distance);

// Restore lag of the highest aut

if(n_lag>n_max_distance) n_lag=0; // Indicates failure

else n_lag--;

if(n_lag==*p_last_periodicity && left_limit_reached) n_lag=0; // Indicates failure

}

*ratio=aut/aut_lag0;

if(*ratio < min_aut_ratio) n_lag=0; // Indicates failure

*p_last_periodicity=n_lag;

}

float rf_rms(float *pn_x, s32 n_size, float *sumsq)

/**

* \brief        Root-mean-square variation

* \par          Details

*               Compute root-mean-square variation for a given series pn_x

*               Robert Fraczkiewicz, 12/25/2017

* \retval       RMS value and raw sum of squares

*/

{

int16_t i;

float r,*pn_ptr;

(*sumsq)=0.0;

for (i=0,pn_ptr=pn_x; i<n_size; ++i,++pn_ptr) {

r=(*pn_ptr);

(*sumsq) += r*r;

}

(*sumsq)/=n_size; // This corresponds to autocorrelation at lag=0

return sqrt(*sumsq);

}

float rf_Pcorrelation(float *pn_x, float *pn_y, s32 n_size)

/**

* \brief        Correlation product

* \par          Details

*               Compute scalar product between *pn_x and *pn_y vectors

*               Robert Fraczkiewicz, 12/25/2017

* \retval       Correlation product

*/

{

int16_t i;

float r,*x_ptr,*y_ptr;

r=0.0;

for (i=0,x_ptr=pn_x,y_ptr=pn_y; i<n_size; ++i,++x_ptr,++y_ptr) {

r+=(*x_ptr)*(*y_ptr);

}

r/=n_size;

return r;

}

## 6. I2C Driver

### I2C driver with status-code checked transactions and multi-byte burst read

#include"header.h"

#define SI ((I2CONSET>>3)&1)

extern volatile u8 sample_buf[6];

void i2c_init(void)

{

// *** I2C Bit freq, setting ***

// Bit freq. = FCLK/(I2SCLL + I2SCLH)

//(I2SCLL + I2SCLH) = 15000000/100000; setting as 100kBps

PINSEL0 |= (1<<4) | (1<<6); //SCL = P0.2 & SDA = P0.3

I2SCLL = (15000000/100000)/2;

I2SCLH = (15000000/100000)/2;

I2CONSET = 1<<6 ;

I2CONCLR = 1<<2;

}

void i2c_burst_read(u32 slave_addr, u32 memory_addr)

{

//u8 result;

s8 i,t=0;

// Generate start condition

I2CONSET = (1<<5); // STA =1;

I2CONCLR = (1<<3); // clearing SI flag

while(SI==0);

if(I2STAT != 0x08) // checking for start condition

{

uart0_tx_string("\r\n ERROR : START condition \r\n");

goto exit ; //need to change to goto exit or somthing

}

I2CONCLR = (1<<5); // STA =0;

// Sending SA+W

I2DAT = slave_addr  ; //SA+W

I2CONCLR = (1<<3); // clearing SI flag

while(SI==0);

if(I2STAT != 0x18) // checking for SA+W,ACK

{

uart0_tx_string("\r\n ERROR : SA + W failed , I2STAT != 0x18 \r\n");

goto exit; //need to change to goto exit or somthing

}

if(I2STAT == 0x20) // checking for SA+W,NOT ACK

{

uart0_tx_string("\r\n ERROR : SA + W failed , NOT ACK \r\n");

goto exit; //need to change to goto exit or somthing

}

// Sending memory addr

I2DAT = memory_addr ;

I2CONCLR = (1<<3); // clearing SI flag

while(SI==0);

if(I2STAT != 0x28)   //  Data/Memory_addr  has been transmitted expected status

{

uart0_tx_string("\r\n ERROR : memory_addr \r\n");

goto exit ; //need to change to goto exit or somthing

}

if(I2STAT == 0x30)   //  Data/Memory_addr  has been transmitted; NOT ACK has been received

{

uart0_tx_string("\r\n ERROR : SA + W failed , NOT ACK \r\n");

goto exit; //need to change to goto exit or somthing

}

// Sending Restart condition

I2CONSET = (1<<5); // STA =1;

I2CONCLR = (1<<3); // clearing SI flag

while(SI==0);

if(I2STAT != 0x10)   //  Reapeted Start condition failed transmitted

{

uart0_tx_string("\r\n ERROR : Restart condition \r\n");

goto exit; //need to change to goto exit or somthing

}

I2CONCLR = (1<<5); // STA =0;

// Sending SA+R

I2DAT = slave_addr^1  ; //SA+R

I2CONCLR = (1<<3); // clearing SI flag

while(SI==0);

if(I2STAT != 0x40) // Expected status

{

uart0_tx_string("\r\n ERROR : SA + W failed, I2STAT != 0x40 \r\n");

goto exit ; //need to change to goto exit or somthing

}

if(I2STAT == 0x48) // checking for SA+W,NOT ACK

{

uart0_tx_string("\r\n ERROR : SA + W failed , NOT ACK \r\n");

goto exit ; //need to change to goto exit or somthing

}

for(i=0;i<5;i++)

{

// Geting data  & sending ACK

I2CONSET=(1<<2); // seting AA=1

I2CONCLR = (1<<3); // clearing SI flag

while(SI==0);

if(I2STAT == 0x58) // Data byte has been received; NOT ACK has been returned

{

uart0_tx_string("\r\n ERROR : Data byte has been received; NOT ACK has been returned \r\n");

goto exit ; //need to change to goto exit or somthing

}

if(I2STAT != 0x50) // Data byte has been received; ACK has been returned

{

uart0_tx_string("\r\n ERROR : ACK \r\n");

goto exit ; //need to change to goto exit or somthing

}

//        uart0_tx_string("\r\n llll Byte = ");

//        uart0_hex( I2DAT);

sample_buf[t++]= I2DAT;

}

//getting last data byte

I2CONCLR = 1<<2;

I2CONCLR = (1<<3); // clearing SI flag

while(SI==0);

if(I2STAT != 0x58) // Data byte has been received; NOT ACK has been returned

{

uart0_tx_string("\r\n ERROR : DATA reception \r\n");

goto exit ; //need to change to goto exit or somthing

}

//        uart0_tx_string("\r\n last Byte = ");

//        uart0_hex( I2DAT);

sample_buf[t++]= I2DAT;

sample_buf[t]= 0;

exit:

I2CONSET = (1<<4);

I2CONCLR = (1<<3);

while((I2CONSET>>4&1) == 1);

//uart0_tx_string("\r\n $$$$$$  READ COMPLETE $$$$$$...\r\n");

}

void i2c_write(u32 slave_addr, u32 memory_addr , u8 data)

{

// Generate start condition

I2CONSET = (1<<5); // STA =1;

I2CONCLR = (1<<3); // clearing SI flag

while(SI==0);

if(I2STAT != 0x08) // checking for start condition

{

uart0_tx_string("\r\n ERROR : START condition \r\n");

goto exit ; //need to change to goto exit or somthing

}

I2CONCLR = (1<<5); // STA =0;

// sending SA+W

I2DAT = slave_addr ; //SA+W

I2CONCLR = (1<<3); // clearing SI flag

while(SI==0);

//				uart0_tx_string("\r\n I2STAT =  ");

//				uart0_intiger(I2STAT);

//				uart0_tx_string("\r\n");

if(I2STAT != 0x18) // checking for SA+W,ACK

{

uart0_tx_string("\r\n ERROR : SA + W failed , I2STAT != 0x18 \r\n");

goto exit ; //need to change to goto exit or somthing

}

if(I2STAT == 0x20) // checking for SA+W,NOT ACK

{

uart0_tx_string("\r\n ERROR : SA + W failed , NOT ACK \r\n");

goto exit ; //need to change to goto exit or somthing

}

// Sending memory addr

I2DAT = memory_addr ;

I2CONCLR = (1<<3); // clearing SI flag

while(SI==0);

if(I2STAT != 0x28)   //  Data/Memory_addr  has been transmitted expected status

{

uart0_tx_string("\r\n ERROR : memory_addr \r\n");

goto exit ; //need to change to goto exit or somthing

}

if(I2STAT == 0x30)   //  Data/Memory_addr  has been transmitted; NOT ACK has been received

{

uart0_tx_string("\r\n ERROR : SA + W failed , NOT ACK \r\n");

goto exit ; //need to change to goto exit or somthing

}

//Send data

I2DAT = data ;

I2CONCLR = (1<<3); // clearing SI flag

while(SI==0);

if(I2STAT != 0x28)   //  Expected status

{

uart0_tx_string("\r\n ERROR : Data , I2STAT != 0x28 \r\n");

goto exit ; //need to change to goto exit or somthing

}

if(I2STAT == 0x30)   //  Data/Memory_addr  has been transmitted; NOT ACK has been received

{

uart0_tx_string("\r\n ERROR : SA + W failed , NOT ACK \r\n");

goto exit ; //need to change to goto exit or somthing

}

exit:

I2CONSET = (1<<4);

I2CONCLR = (1<<3);

while((I2CONSET>>4&1) == 1);

//uart0_tx_string("\r\n !!!!!!  Write COMPLETE !!!!!...\r\n");

}

u8 i2c_read(u32 slave_addr, u32 memory_addr)

{

u8 result;

// Generate start condition

I2CONSET = (1<<5); // STA =1;

I2CONCLR = (1<<3); // clearing SI flag

while(SI==0);

I2CONCLR = (1<<5); // STA =0;

// Sending SA+W

I2DAT = slave_addr  ; //SA+W

I2CONCLR = (1<<3); // clearing SI flag

while(SI==0);

if(I2STAT != 0x18) // checking for SA+W,ACK

{

uart0_tx_string("\r\n ERROR : SA + W failed , I2STAT != 0x18 \r\n");

goto exit; //need to change to goto exit or somthing

}

if(I2STAT == 0x20) // checking for SA+W,NOT ACK

{

uart0_tx_string("\r\n ERROR : SA + W failed , NOT ACK \r\n");

goto exit; //need to change to goto exit or somthing

}

// Sending memory addr

I2DAT = memory_addr ;

I2CONCLR = (1<<3); // clearing SI flag

while(SI==0);

if(I2STAT != 0x28)   //  Data/Memory_addr  has been transmitted expected status

{

uart0_tx_string("\r\n ERROR : memory_addr \r\n");

goto exit ; //need to change to goto exit or somthing

}

if(I2STAT == 0x30)   //  Data/Memory_addr  has been transmitted; NOT ACK has been received

{

uart0_tx_string("\r\n ERROR : SA + W failed , NOT ACK \r\n");

goto exit; //need to change to goto exit or somthing

}

// Sending Restart condition

I2CONSET = (1<<5); // STA =1;

I2CONCLR = (1<<3); // clearing SI flag

while(SI==0);

if(I2STAT != 0x10)   //  Reapeted Start condition failed transmitted

{

uart0_tx_string("\r\n ERROR : Restart condition \r\n");

goto exit; //need to change to goto exit or somthing

}

I2CONCLR = (1<<5); // STA =0;

// Sending SA+R

I2DAT = slave_addr^1  ; //SA+R

I2CONCLR = (1<<3); // clearing SI flag

while(SI==0);

if(I2STAT != 0x40) // Expected status

{

uart0_tx_string("\r\n ERROR : SA + W failed, I2STAT != 0x40 \r\n");

goto exit ; //need to change to goto exit or somthing

}

if(I2STAT == 0x48) // checking for SA+W,NOT ACK

{

uart0_tx_string("\r\n ERROR : SA + W failed , NOT ACK \r\n");

goto exit ; //need to change to goto exit or somthing

}

// Geting data

I2CONCLR = 1<<2;

I2CONCLR = (1<<3); // clearing SI flag

while(SI==0);

if(I2STAT != 0x58) // checking for 0x58 status

{

uart0_tx_string("\r\n ERROR : DATA reception \r\n");

goto exit ; //need to change to goto exit or somthing

}

result = I2DAT;

exit:

//        I2CONSET = (1<<4);

//        I2CONCLR = (1<<3);

//        delay_ms(100);

I2CONSET = (1<<4);

I2CONCLR = (1<<3);

while((I2CONSET>>4&1) == 1);

//uart0_tx_string("\r\n $$$$$$  READ COMPLETE $$$$$$...\r\n");

return result;

}

## 7. MAX30102 Sensor Driver

### Sensor initialization, temperature reading, and warning handler

#include"header.h"

// MAX30102

volatile u8 sample_buf[6];

extern u32 ir_buffer[MY_BUFFER_SIZE], red_buffer[MY_BUFFER_SIZE];

static u32 index=0;

/////// WARNING ////////

void warning (void)

{

IOCLR0 =  WARNING_LED;

IOCLR0 = BUZZER;

lcd_cmd(0x01);

lcd_cmd(0x80);

lcd_str("WARNING");

//can use an switch interrupt here to reset from this while(sw==0); inside isr flag=1;

}

void MAX_init(void)

{

uart0_tx_string("\r\n IN MAX_init...\r\n");

// mode configartion 0x09

i2c_write(0xAE,0x09,0x40);

do

{

uart0_tx_string("\r\n Checking power ready \r\n");

uart0_tx_string("\r\n Power ready = ");

uart0_hex(i2c_read(0xae,0x00));

}while(i2c_read(0xae,0x09) & 0x40);

// FIFO configaration(0X08)

//i2c_write(0xAE,0x08,0xB0);   //Sample averaging = 32 samples, FIFO rollover enabled

i2c_write(0xAE,0x08,0x00);   //Sample averaging = 0 samples, FIFO rollover disabled

// spo2 configaration 0x0A

i2c_write(0xAE,0x0A,0x27);

// LED pulse amplitude ( 0x0C - 0X0D)

i2c_write(0xAE,0x0C,0x3F);

i2c_write(0xAE,0x0D,0x24);

i2c_write(0xAE,0x09,0x03);

// clearing the FIFO_WR_PTR, OVF_COUNTER, and FIFO_RD_PTR

i2c_write(0xAE,0x04,0x00);

//uart0_tx_string("\r\n read_FIFO_WR_PTR = ");

//uart0_hex(read_FIFO_WR_PTR());

//

//uart0_tx_string("\r\n read_OVR_COUNTER = ");

i2c_write(0xAE,0x05,0x00);

//uart0_hex(read_OVR_COUNTER());

i2c_write(0xAE,0x06,0x00);

//uart0_tx_string("\r\n read_FIFO_RD_PTR = ");

//uart0_hex(read_FIFO_RD_PTR());

//

//uart0_tx_string("\r\n Power ready = ");

//uart0_hex(i2c_read(0xae,0x00));

// Interrupt ******

i2c_write(0xAE,0x02,0xE0);

i2c_write(0xAE,0x03,0x02);

uart0_tx_string("\r\nMAX_init completed...\r\n");

}

u8 sp02_sample_temp(void)

{

//u8 wr_ptr,rd_ptr , no_of_available_samples,full=0;

u32 red_raw, IR_raw ;

u8 i=0;

//uart0_tx_string("\r\n read_FIFO_WR_PTR = ");

//uart0_intiger(read_FIFO_WR_PTR());

//

//uart0_tx_string("\r\n read_OVR_COUNTER = ");

//uart0_intiger(read_OVR_COUNTER());

//

//uart0_tx_string("\r\n read_FIFO_RD_PTR = ");

//uart0_intiger(read_FIFO_RD_PTR());

while(i<32)

{

i2c_burst_read(0xAE,0x07);

red_raw = (sample_buf[0] & 0x03)<<16 | (sample_buf[1]<<8) | sample_buf[2] ;

IR_raw = (sample_buf[3] & 0x03)<<16 | (sample_buf[4]<<8) | sample_buf[5] ;

uart0_tx_string("\r\n red_raw : ");

uart0_intiger(red_raw);

uart0_tx_string("  | IR_raw : ");

uart0_intiger(IR_raw);

ir_buffer[index]=IR_raw;

red_buffer[index]=red_raw;

index++;

if(index==(MY_BUFFER_SIZE))

{

uart0_tx_string("\r\n buffer =400 \r\n ");

index=0;

return 1;

}

i++;

}

//uart0_tx_string("\r\n index : ");

//uart0_intiger(index);

return 0;

}

u8 sp02_sample(void)

{

u8 wr_ptr,rd_ptr , no_of_available_samples,full=0;

u32 red_raw, IR_raw ;

uart0_tx_string("\r\n ******** in sample... ************\r\n");

wr_ptr = read_FIFO_WR_PTR();

rd_ptr = read_FIFO_RD_PTR();

if( wr_ptr > rd_ptr)

{

uart0_tx_string("\r\n Inside if \r\n ");

no_of_available_samples =  wr_ptr - rd_ptr;

}

else if ( wr_ptr < rd_ptr )

{

uart0_tx_string("\r\n Inside else if \r\n ");

no_of_available_samples =  32 + wr_ptr - rd_ptr;

}

else

{

uart0_tx_string("\r\nINT1 = ");

uart0_hex(i2c_read(0xAE, 0x00));

uart0_tx_string("\r\nWR = ");

uart0_intiger(read_FIFO_WR_PTR());

uart0_tx_string("\r\nRD = ");

uart0_intiger(read_FIFO_RD_PTR());

uart0_tx_string("\r\n Inside else \r\n ");

return 0;

}

uart0_tx_string("\r\n wr_ptr = ");

uart0_intiger(wr_ptr);

uart0_tx_string("\r\n rd_ptr = ");

uart0_intiger(rd_ptr);

uart0_tx_string("\r\n no. of samples = ");

uart0_intiger(no_of_available_samples);

while(no_of_available_samples)

{

i2c_burst_read(0xAE,0x07);

red_raw = (sample_buf[0] & 0x03)<<16 | (sample_buf[1]<<8) | sample_buf[2] ;

IR_raw = (sample_buf[3] & 0x03)<<16 | (sample_buf[4]<<8) | sample_buf[5] ;

//   uart0_tx_string("\r\n red_raw : ");

//   uart0_intiger(red_raw);

//   uart0_tx_string("  | IR_raw : ");

//   uart0_intiger(IR_raw);

ir_buffer[index]=IR_raw;

red_buffer[index]=red_raw;

index++;

if(index>=(MY_BUFFER_SIZE))

{

uart0_tx_string("\r\n buffer =600 \r\n ");

index=0;

full =1;

}

no_of_available_samples -- ;

}

uart0_tx_string("\r\n index : ");

uart0_intiger(index);

return full;

}

u8 temp(void)

{

u8 temp_fraction;

i2c_write(0xAE,0x21,0x01);

while( i2c_read(0xAE,0x21) &1);

temp_fraction = i2c_read(0xAE,0x20);

return ( i2c_read(0xAE,0x1F) + ( temp_fraction & 0x0F) );

}

u8 read_FIFO_WR_PTR(void)

{

u8 res;

res = i2c_read(0xAE,0x04);

return   res;// & 0x0f;

}

u8 read_OVR_COUNTER(void)

{

u8 res;

res = i2c_read(0xAE,0x05);

return   res ;//& 0x0f;

}

u8 read_FIFO_RD_PTR(void)

{

u8 res;

res = i2c_read(0xAE,0x06);

return   res ;//& 0x0f;

}

u32 average(u32 * b)

{

u32 av=0,i;

for( i=0;i<MY_BUFFER_SIZE;i++)

{

av=av+b[i];

}

return av/MY_BUFFER_SIZE;

}

## 8. Main Application —  ThingSpeak Upload

### Main loop with HR/SpO2 display, and ThingSpeak data upload

#include"header.h"

volatile u8 timer_flag;

#define LED 1<<3

//u32 ir_buffer[MY_BUFFER_SIZE], red_buffer[MY_BUFFER_SIZE];

u32 ir_buffer[100] = {

90000, 90040, 90080, 90120, 90160, 90200, 90240, 90280, 90320, 90360,

90400, 90440, 90480, 90520, 90560, 90600, 90640, 90680, 90720, 90760,

90800, 90840, 90880, 90920, 90960, 91000, 90960, 90920, 90880, 90840,

90800, 90760, 90720, 90680, 90640, 90600, 90560, 90520, 90480, 90440,

90400, 90360, 90320, 90280, 90240, 90200, 90160, 90120, 90080, 90040,

90000, 90040, 90080, 90120, 90160, 90200, 90240, 90280, 90320, 90360,

90400, 90440, 90480, 90520, 90560, 90600, 90640, 90680, 90720, 90760,

90800, 90840, 90880, 90920, 90960, 91000, 90960, 90920, 90880, 90840,

90800, 90760, 90720, 90680, 90640, 90600, 90560, 90520, 90480, 90440,

90400, 90360, 90320, 90280, 90240, 90200, 90160, 90120, 90080, 90040

};

u32 red_buffer[100] = {

70000, 70020, 70040, 70060, 70080, 70100, 70120, 70140, 70160, 70180,

70200, 70220, 70240, 70260, 70280, 70300, 70320, 70340, 70360, 70380,

70400, 70420, 70440, 70460, 70480, 70500, 70480, 70460, 70440, 70420,

70400, 70380, 70360, 70340, 70320, 70300, 70280, 70260, 70240, 70220,

70200, 70180, 70160, 70140, 70120, 70100, 70080, 70060, 70040, 70020,

70000, 70020, 70040, 70060, 70080, 70100, 70120, 70140, 70160, 70180,

70200, 70220, 70240, 70260, 70280, 70300, 70320, 70340, 70360, 70380,

70400, 70420, 70440, 70460, 70480, 70500, 70480, 70460, 70440, 70420,

70400, 70380, 70360, 70340, 70320, 70300, 70280, 70260, 70240, 70220,

70200, 70180, 70160, 70140, 70120, 70100, 70080, 70060, 70040, 70020

};

int main()

{

//u32 i;

float spo2;

float ratio;

float correlation;

s32 heart_rate , temp_sensor , count;

s8 hr_valid, spo2_valid ;

IODIR0 = WARNING_LED;

uart0_init(9600);

i2c_init();

MAX_init();

adc_init();

uart0_tx_string("\r\n Started \r\n "	);

lcd_cmd(0x01);

lcd_cmd(0x80);

lcd_str("Starting....");

////**** Set wifi return status , if not connected try again , set try again limit ( only connected then move forward) .. etc

lcd_cmd(0x01);

lcd_cmd(0x80);

lcd_str("Searching WIFI");

count = 0;

while((wifi_start())==0)

{

if( ++count == 10)

{

lcd_cmd(0x01);

lcd_cmd(0x80);

lcd_str("ERROR : WIFI");

lcd_cmd(0xC0);

lcd_str("Not Connected");

break;

}

}

if(count < 10)

{

lcd_cmd(0x01);

lcd_cmd(0x80);

lcd_str("WIFI CONECTED");

lcd_cmd(0xC0);

lcd_str("SUCCESSFUL");

}

/////////////////////////////////////////////////////////////////////////////////////////

while(1)

{

if(sp02_sample_temp())

{

if (ir_buffer[0] < 50000)

{

uart0_tx_string("\r\n Sensor not in use. Please place your finger.");

lcd_cmd(0xC0);

lcd_str("NO SENSOR");

}

else

{

rf_heart_rate_and_oxygen_saturation( ir_buffer, BUFFER_SIZE, red_buffer, &spo2, &spo2_valid, &heart_rate, &hr_valid, &ratio, &correlation );

/// Heart rate

if(hr_valid)

{

uart0_tx_string("\r\n HR =  "	);

uart0_intiger(heart_rate); //printf("HR = %d", heart_rate);

lcd_cmd(0x80);

lcd_str("HR : ");

lcd_integer(heart_rate);

}

else

{

uart0_tx_string("\r\n Invalid HR \r\n "	);

warning();

lcd_cmd(0xC0);

lcd_str("INVALID HR");

}

/// spo2

if(spo2_valid)

{

uart0_tx_string("\r\n SPO2 =  "	);

uart0_float(spo2); //printf("SpO2 = %.1f", spo2);

lcd_cmd(0xC0);

lcd_str("SPO2 : ");

lcd_float(spo2);

}

else

{

uart0_tx_string("\r\n Invalid SPO2 \r\n "	);

warning();

lcd_cmd(0xC0);

lcd_str("INVALID SPO2");

}

// sending data to THINKSPEAK

count=0;

if(timer_flag)

{

timer_flag=0;

if((send_to_Thinkspeak(heart_rate,  spo2 ,  temp_sensor)==0))

{

uart0_tx_string("\r\n !!!!!! ERROR : Unable to send data to thinkspeak !!!!!!!\r\n "	);

}

}

}

}

delay_ms(1000);

}

}

## 9. LCD Driver

### 4-bit LCD data/command routines, string, integer, and float display functions

#include"header.h"

void lcd_data( unsigned char data)

{ //higher nibble

unsigned int t;

IOCLR1=0xfe<<16;

t=(data & 0xf0)<<16;

IOSET1=t;

IOSET1=1<<17;

IOCLR1=1<<18;

IOSET1=1<<19;

delay_ms(2);

IOCLR1=1<<19;

//Lower nibble

IOCLR1=0xfe<<16;

t=(data & 0x0f)<<20;

IOSET1=t;

IOSET1=1<<17;

IOCLR1=1<<18;

IOSET1=1<<19;

delay_ms(2);

IOCLR1=1<<19;

}

void lcd_cmd( unsigned char cmd)

{

unsigned int t;

IOCLR1=0xfe<<16;

t=(cmd & 0xf0)<<16;

IOSET1=t;

IOCLR1=1<<17;

IOCLR1=1<<18;

IOSET1=1<<19;

delay_ms(2);

IOCLR1=1<<19;

IOCLR1=0xfe<<16;

t=(cmd & 0x0f)<<20;

IOSET1=t;

IOCLR1=1<<17;

IOCLR1=1<<18;

IOSET1=1<<19;

delay_ms(2);

IOCLR1=1<<19;

}

void lcd_init(void)

{

IODIR1=0xfe<<16;

PINSEL2|=0x0;

lcd_cmd(0x02);

lcd_cmd(0x28);

lcd_cmd(0x0e);

lcd_cmd(0x01);

//lcd_cmd(0x0f);

}

void lcd_str(s8 *p)

{

while(*p)

{

lcd_data(*p);

p++;

}

}

int str_len(char *p)

{

int i;

for(i=0;*p;p++,i++);

return i;

}

void float1(float f)

{

int num,dec,i,j=0,a[20],t,s;

num=(int)f;

dec=(f-num)*1000000;

for(i=dec;i;i=i/10,j++);

for(i=0;j;j--,i++,dec=dec/10)

{

a[i]=dec%10 + 48;

}

a[i]='.';

t=++i;

for(i=num;i;i=i/10,j++){}

for(i=t,s=j;j;j--,i++,num=num/10)

{

a[i]=num%10 + 48;

}

for(i=6+s;i>=0;i--)

lcd_data(a[i]);

}

void lcd_integer(int num)

{

int i,a[10];

if(num==0)

lcd_data('0');

if(num<0)

{

num=-num;

lcd_data('-');

}

for(i=0;num;num=num/10,i++)

{

a[i]=num%10 +48;

}

for(i=i-1;i>=0;i--)

lcd_data(a[i]);

}

void lcd_float(float num)

{

u32 intnum,decinum;

s32 i=0;

u8 a[10];

if(num<0)

{

uart0_tx_string("\r\n LCD : Num is -ve\r\n");

//str_len("Num is -ve");

return;

}

if(num==0)

{

lcd_data('0');

return;

}

intnum=num;

decinum=((num-intnum)*10);

if(intnum==0)

{

lcd_data('0');

}

else {

while(intnum)

{

a[i]=intnum%10;

intnum/=10;

i++;

}

while(i>0)

{

i--;

lcd_data(a[i]+48);

}

}

lcd_data('.');

if(decinum==0)

{

lcd_data('0');

return;

}

else {

while(decinum)

{

a[i]=decinum%10;

decinum/=10;

i++;

}

while(i>0)

{

i--;

lcd_data(a[i]+48);

}

}

}

## 10. UART0 Driver

### UART0 init, RX/TX, string/float/integer/hex transmit and receive helpers

#include"header.h"

void uart0_init(unsigned int baud)

{

unsigned int pclk,d;

if(VPBDIV==0)

pclk=15000000;

else if(VPBDIV==1)

pclk=60000000;

else if(VPBDIV==2)

pclk=30000000;

PINSEL0 |=0x05;

d=pclk/(16*baud);

U0LCR=0x83;

U0DLL=d&0xff;

U0DLM=(d>>8)&0xff;

U0LCR=0x03;

}

void uart0_rx_buffer_ESPwifi(u8 *buf)

{

int i=0;

while(1)

{

buf[i]=uart0_rx();

if(i>3)

{

if(buf[i]=='\n' &&  buf[i-1]=='\r')

{

break;

}

}

i++;

}

buf[i]=0;

}

void uart0_tx(unsigned char data)

{

U0THR=data;

while(((U0LSR>>5)&1)==0);

}

unsigned char uart0_rx(void)

{

while(((U0LSR>>0)&1)==0);

return U0RBR;

}

void uart0_tx_string(char *p)

{

while(*p)

{

uart0_tx(*p);

//while((U0LSR>>5&1)==0);

p++;

}

}

void uart0_rx_string(unsigned char * s)

{

int i=0;

unsigned char t ;

while((t=uart0_rx())!='\r')

{

uart0_tx(t);

s[i]=t;

i++;

}

s[i]=0;

}

void uart0_float(float num)

{

u32 intnum,decinum;

s32 i=0;

u8 a[7];

if(num<0)

{

uart0_tx_string("\r\n Num is -ve\r\n");

return;

}

if(num==0)

{

uart0_tx('0');

return;

}

intnum=num;

decinum=((num-intnum)*10);

if(intnum==0)

{

uart0_tx('0');

}

else {

while(intnum)

{

a[i]=intnum%10;

intnum/=10;

i++;

}

while(i>0)

{

i--;

uart0_tx(a[i]+48);

}

}

uart0_tx('.');

if(decinum==0)

{

uart0_tx('0');

return;

}

else {

while(decinum)

{

a[i]=decinum%10;

decinum/=10;

i++;

}

while(i>0)

{

i--;

uart0_tx(a[i]+48);

}

}

}

void uart0_intiger(s32 num)

{

u32 a[10];

s32 i=0;

if(num < 0 || num == 0)

{

if( num == 0)

{

uart0_tx('0');

return;

}

uart0_tx_string("\r\n Negative number \r\n");

return;

}

while(num)

{

a[i]=num%10;

num/=10;

i++;

}

while(i)

{

--i;

uart0_tx(a[i]+48);

}

}

u32 uart0_rx_string_decimal(void)

{

s32 i=0;

u32 d=0;

u8 t,s[32];

while((t=uart0_rx())!='\r')

{

uart0_tx(t);

s[i]=t;

i++;

}

s[i]=0;

i=0;

while(s[i])

{

d=(d*10)+(s[i]-48);

i++;

}

return d;

}

void uart0_hex(u8 num) //for 1 byte

{

u8 t;

t = (num>>4&0x0f);

if(t<10)

{

uart0_tx(t+48);

}

else

{

uart0_tx(t-10+65);

}

t = (num&0x0f);

if(t<10)

{

uart0_tx(t+48);

}

else

{

uart0_tx(t-10+65);

}

}

## 11. Saline Driver

#include<lpc21xx.h>

#include"header.h"

#define TRIG_PIN (1<<20)

#define ECHO_PIN (1<<23)

int trig;

void ultrasonic_init(void)

{

IODIR0|=TRIG_PIN;

IODIR0&=~(ECHO_PIN);

T0TCR=2;

T0PR=59;

T0TCR=0;

}

unsigned int distance(void)

{

unsigned int time=0;

unsigned int dis=0;

unsigned int val=0;

IOSET0=TRIG_PIN;

delay_us(10);

IOCLR0=TRIG_PIN;

uart0_str("abcd \r\n ");

while ((IOPIN0 & (ECHO_PIN))==0);

uart0_str("fghi\r\n ");

T0TCR=0X02;

T0TC=0;

T0TCR=0X01;

uart0_str("pqrs \r\n ");

val=(IOPIN0&(ECHO_PIN));

uart0_str("\r\nval:");

uart0_int(val);

while (!(IOPIN0 & (ECHO_PIN)));

uart0_str("mnop \r\n ");

T0TCR=0X00;

time=T0TC;

dis=time/58;

//dis=((c*2.0)/58)/30.48;

return dis;

}

## 12. External Interrupt

#include"header.h"

//extern u32 flag;

void eint0_handler()__irq

{

//emergence();

EXTINT=1;

//flag=1;

VICVectAddr=0;

}

void config_eint0()

{

PINSEL1|=0x1;

EXTMODE=1;

EXTPOLAR=0;

VICIntSelect=0;

VICVectCntl0=14|(1<<5);

VICVectAddr0=(u32)eint0_handler;

VICIntEnable=1<<14;

}

## 13. DTH11 driver

#include "header.h"

unsigned char RH_Int;

unsigned char RH_Dec;

unsigned char Temp_Int;

unsigned char Temp_Dec;

unsigned char CheckSum;

unsigned int timeout;

#define DHT_PIN 11

void DHT11_init(void)

{

IODIR0 |= (1<<DHT_PIN);

IOSET0 |= (1<<DHT_PIN);

}

unsigned char DHT11_Start(void)

{

/* Configure as Output */

IODIR0 |= (1<<DHT_PIN);

/* Pull Low for 18ms */

IOCLR0 = (1<<DHT_PIN);

delay_ms(18);

/* Pull High for 30us */

IOSET0 = (1<<DHT_PIN);

delay_us(30);

/* Configure as Input */

IODIR0 &= ~(1<<DHT_PIN);

timeout = 10000;

while((IOPIN0 & (1<<DHT_PIN)) && timeout--);

if(timeout == 0) return 0;

timeout = 10000;

while((!(IOPIN0 & (1<<DHT_PIN))) && timeout--);

if(timeout == 0) return 0;

timeout = 10000;

while((IOPIN0 & (1<<DHT_PIN)) && timeout--);

if(timeout == 0) return 0;

return 1;

}

unsigned char DHT11_ReadByte(void)

{

unsigned char i;

unsigned char data = 0;

for(i=0;i<8;i++)

{

timeout = 1000;

while((!(IOPIN0 & (1<<DHT_PIN))) && timeout--);

if(timeout == 0)

return 0xFF;

delay_us(40);

if(IOPIN0 & (1<<DHT_PIN))

data = (data<<1) | 1;

else

data = (data<<1);

timeout = 1000;

while((IOPIN0 & (1<<DHT_PIN)) && timeout--);

if(timeout == 0)

return 0xFF;

}

return data;

## 13. Delay

#include"header.h"

void delay_ms(u32 ms)

{

T0PR=15000-1;

T0PC=0;

T0TC=0;

T0TCR=1;

while(T0TC<ms);

T0TCR=0;

}

void delay_sec(u32 sec)

{

T0PR=15000000-1;

T0PC=0;

T0TC=0;

T0TCR=1;

while(T0TC<sec);

T0TCR=0;

}

void delay_us(u32 us)

{

T0PR=15-1;

T0PC=0;

T0TC=0;

T0TCR=1;

while(T0TC<us);

T0TCR=0;

}